Lab – Configuring EtherChannel

Topology

Addressing Table

| Device | Interface | IP Address | Subnet Mask |
| --- | --- | --- | --- |
| S1 | VLAN 99 | 192.168.99.11 | 255.255.255.0 |
| S2 | VLAN 99 | 192.168.99.12 | 255.255.255.0 |
| S3 | VLAN 99 | 192.168.99.13 | 255.255.255.0 |
| PC-A | NIC | 192.168.10.1 | 255.255.255.0 |
| PC-B | NIC | 192.168.10.2 | 255.255.255.0 |
| PC-C | NIC | 192.168.10.3 | 255.255.255.0 |

Objectives

Part 1: Configure Basic Switch Settings

Part 2: Configure PAgP

Part 3: Configure LACP

Background / Scenario

Link aggregation allows the creation of logical links that are comprised of two or more physical links. This provides increased throughput beyond using only one physical link. Link aggregation also provides redundancy if one of the links fails.

In this lab, you will configure EtherChannel, a form of link aggregation used in switched networks. You will configure EtherChannel using Port Aggregation Protocol (PAgP) and Link Aggregation Control Protocol (LACP).

Note: PAgP is a Cisco-proprietary protocol that you can only run on Cisco switches and on switches that are licensed vendors to support PAgP. LACP is a link aggregation protocol that is defined by IEEE 802.3ad, and it is not associated with any specific vendor.

LACP allows Cisco switches to manage Ethernet channels between switches that conform to the 802.3ad protocol. You can configure up to 16 ports to form a channel. Eight of the ports are in active mode and the other eight are in standby mode. When any of the active ports fail, a standby port becomes active. Standby mode works only for LACP, not for PAgP.

Note: The switches used with CCNA hands-on labs are Cisco Catalyst 2960s with Cisco IOS Release 15.0(2) (lanbasek9 image). Other switches and Cisco IOS versions can be used. Depending on the model and Cisco IOS version, the commands available and output produced might vary from what is shown in the labs.

Note: Make sure that the switches have been erased and have no startup configurations. If you are unsure, contact your instructor.

Required Resources

3 Switches (Cisco 2960 with Cisco IOS Release 15.0(2) lanbasek9 image or comparable)

3 PCs (Windows 7, Vista, or XP with terminal emulation program, such as Tera Term)

Console cables to configure the Cisco IOS devices via the console ports

Ethernet cables as shown in the topology

Configure Basic Switch Settings

In Part 1, you will set up the network topology and configure basic settings, such as the interface IP addresses, device access, and passwords.

Cable the network as shown in the topology.

Attach the devices as shown in the topology diagram, and cable as necessary.

Initialize and reload the switches.

Configure basic settings for each switch.

Disable DNS lookup.

Configure the device name as displayed in the topology.

Encrypt plain text passwords.

Create a MOTD banner warning users that unauthorized access is prohibited.

Assign class as the encrypted privileged EXEC mode password.

Assign cisco as the console and vty password and enable login.

Configure logging synchronous to prevent console message from interrupting command entry.

Shut down all switchports except the ports connected to PCs.

Configure VLAN 99 and name it Management.

Configure VLAN 10 and name it Staff.

Configure the switch ports with attached hosts as access ports in VLAN 10.

Assign the IP addresses according to the Addressing Table.

Copy the running configuration to startup configuration.

Configure the PCs.

Assign IP addresses to the PCs according to the Addressing Table.

Configure PAgP

PAgP is a Cisco proprietary protocol for link aggregation. In Part 2, a link between S1 and S3 will be configured using PAgP.

Configure PAgP on S1 and S3.

For a link between S1 and S3, configure the ports on S1 with PAgP desirable mode and the ports on S3 with PAgP auto mode. Enable the ports after PAgP modes have been configured.

S1(config)# interface range f0/3-4

S1(config-if-range)# channel-group 1 mode desirable

Creating a port-channel interface Port-channel 1

S1(config-if-range)# no shutdown

S3(config)# interface range f0/3-4

S3(config-if-range)# channel-group 1 mode auto

Creating a port-channel interface Port-channel 1

S3(config-if-range)# no shutdown

*Mar  1 00:09:12.792: %LINK-3-UPDOWN: Interface FastEthernet0/3, changed state to up

*Mar  1 00:09:12.792: %LINK-3-UPDOWN: Interface FastEthernet0/4, changed state to up

S3(config-if-range)#

*Mar  1 00:09:15.384: %LINEPROTO-5-UPDOWN: Line protocol on Interface FastEthernet0/3, changed state to up

*Mar  1 00:09:16.265: %LINEPROTO-5-UPDOWN: Line protocol on Interface FastEthernet0/4, changed state to up

S3(config-if-range)#

*Mar  1 00:09:16.357: %LINK-3-UPDOWN: Interface Port-channel1, changed state to up

*Mar  1 00:09:17.364: %LINEPROTO-5-UPDOWN: Line protocol on Interface Port-channel1, changed state to up

*Mar  1 00:09:44.383: %LINEPROTO-5-UPDOWN: Line protocol on Interface Vlan1, changed state to up

Examine the configuration on the ports.

Currently the F0/3, F0/4, and Po1 (Port-channel1) interfaces on both S1 and S3 are in access operational mode with the administrative mode in dynamic auto. Verify the configuration using the show run interface interface-id and show interfaces interface-id switchport commands, respectively. The example configuration outputs for F0/3 on S1 are as follows:

S1# show run interface f0/3

Building configuration...

Current configuration : 103 bytes

!

interface FastEthernet0/3

channel-group 1 mode desirable

S1# show interfaces f0/3 switchport

Name: Fa0/3

Switchport: Enabled

Administrative Mode: dynamic auto

Operational Mode: static access (member of bundle Po1)

Administrative Trunking Encapsulation: dot1q

Operational Trunking Encapsulation: native

Negotiation of Trunking: On

Access Mode VLAN: 1 (default)

Trunking Native Mode VLAN: 1 (default)

Administrative Native VLAN tagging: enabled

Voice VLAN: none

Administrative private-vlan host-association: none

Administrative private-vlan mapping: none

Administrative private-vlan trunk native VLAN: none

Administrative private-vlan trunk Native VLAN tagging: enabled

Administrative private-vlan trunk encapsulation: dot1q

Administrative private-vlan trunk normal VLANs: none

Administrative private-vlan trunk associations: none

Administrative private-vlan trunk mappings: none

Operational private-vlan: none

Trunking VLANs Enabled: ALL

Pruning VLANs Enabled: 2-1001

Capture Mode Disabled

Capture VLANs Allowed: ALL

Protected: false

Unknown unicast blocked: disabled

Unknown multicast blocked: disabled

Appliance trust: none

Verify that the ports have been aggregated.

S1# show etherchannel summary

Flags:  D - down        P - bundled in port-channel

I - stand-alone s - suspended

H - Hot-standby (LACP only)

R - Layer3      S - Layer2

U - in use      f - failed to allocate aggregator

M - not in use, minimum links not met

u - unsuitable for bundling

w - waiting to be aggregated

d - default port

Number of channel-groups in use: 1

Number of aggregators:           1

Group  Port-channel  Protocol    Ports

------+-------------+-----------+-----------------------------------------------

1      Po1(SU)         PAgP      Fa0/3(P)    Fa0/4(P)

S3# show etherchannel summary

Flags:  D - down        P - bundled in port-channel

I - stand-alone s - suspended

H - Hot-standby (LACP only)

R - Layer3      S - Layer2

U - in use      f - failed to allocate aggregator

M - not in use, minimum links not met

u - unsuitable for bundling

w - waiting to be aggregated

d - default port

Number of channel-groups in use: 1

Number of aggregators:           1

Group  Port-channel  Protocol    Ports

------+-------------+-----------+-----------------------------------------------

1      Po1(SU)         PAgP      Fa0/3(P)    Fa0/4(P)

What do the flags, SU and P, indicate in the Ethernet summary?

The S flag indicates that the port-channel is a Layer 2 Etherchannel. The U flag indicates that the EtherChannel is up. So when you see SU it means the Layer 2 is up and running. The P flag indicates that the ports are bundled

Configure trunk ports.

After the ports have been aggregated, commands applied at the port channel interface affect all the links that were bundled together. Manually configure the Po1 ports on S1 and S3 as trunk ports and assign them to native VLAN 99.

S1(config)# interface port-channel 1

S1(config-if)# switchport mode trunk

S1(config-if)# switchport trunk native vlan 99

S3(config)# interface port-channel 1

S3(config-if)# switchport mode trunk

S3(config-if)# switchport trunk native vlan 99

Verify that the ports are configured as trunk ports.

Issue the show run interface interface-id commands on S1 and S3. What commands are listed for F0/3 and F0/4 on both switches? Compare the results to the running configuration for the Po1 interface? Record your observation.

- Switchport trunk native vlan 99Switchport mode trunkThe commands affect the individual links in the bundle when the trunk commands are applied to the Etherchannel

Issue the show interfaces trunk and show spanning-tree commands on S1 and S3. What trunk port is listed? What is the native VLAN? What is concluding result from the output?

- The trunk port listed is Po#. The native 3LA+ is 88 after the links are bundled only the aggregated interfaces is listed in some show commands

From the show spanning-tree output, what is port cost and port priority for the aggregated link?

The port cost for Po1 is 12 the port priority is 128

Configure LACP

LACP is an open source protocol for link aggregation developed by the IEEE. In Part 3, the link between S1 and S2, and the link between S2 and S3 will be configured using LACP. Also, the individual links will be configured as trunks before they are bundled together as EtherChannels.

Configure LACP between S1 and S2.

S1(config)# interface range f0/1-2

S1(config-if-range)# switchport mode trunk

S1(config-if-range)# switchport trunk native vlan 99

S1(config-if-range)# channel-group 2 mode active

Creating a port-channel interface Port-channel 2

S1(config-if-range)# no shutdown

S2(config)# interface range f0/1-2

S2(config-if-range)# switchport mode trunk

S2(config-if-range)# switchport trunk native vlan 99

S2(config-if-range)# channel-group 2 mode passive

Creating a port-channel interface Port-channel 2

S2(config-if-range)# no shutdown

Verify that the ports have been aggregated.

What protocol is Po2 using for link aggregation? Which ports are aggregated to form Po2? Record the command used to verify.

Po2 is using LACP and f0/1 and F02 are aggregates from Po2

Configure LACP between S2 and S3.

Configure the link between S2 and S3 as Po3 and use LACP as the link aggregation protocol.

S2(config)# interface range f0/3-4

S2(config-if-range)# switchport mode trunk

S2(config-if-range)# switchport trunk native vlan 99

S2(config-if-range)# channel-group 3 mode active

Creating a port-channel interface Port-channel 3

S2(config-if-range)# no shutdown

S3(config)# interface range f0/1-2

S3(config-if-range)# switchport mode trunk

S3(config-if-range)# switchport trunk native vlan 99

S3(config-if-range)# channel-group 3 mode passive

Creating a port-channel interface Port-channel 3

S3(config-if-range)# no shutdown

Verify that the EtherChannel has formed.

Verify end-to-end connectivity.

Verify that all devices can ping each other within the same VLAN. If not, troubleshoot until there is end-to-end connectivity.

Note: It may be necessary to disable the PC firewall to ping between PCs.

Reflection

What could prevent EtherChannels from forming?

Configuration mismatch, such as trunk port on one end and access port on the other end different aggregation protocols and different speed/duplex would prevent the formation of EtherChannel