Lab – Configuring Basic EIGRP for IPv4

Topology

Addressing Table

| Device | Interface | IP Address | Subnet Mask | Default Gateway |
| --- | --- | --- | --- | --- |
| R1 | G0/0 | 192.168.1.1 | 255.255.255.0 | N/A |
|  | S0/0/0 (DCE) | 10.1.1.1 | 255.255.255.252 | N/A |
|  | S0/0/1 | 10.3.3.1 | 255.255.255.252 | N/A |
| R2 | G0/0 | 192.168.2.1 | 255.255.255.0 | N/A |
|  | S0/0/0 | 10.1.1.2 | 255.255.255.252 | N/A |
|  | S0/0/1 (DCE) | 10.2.2.2 | 255.255.255.252 | N/A |
| R3 | G0/0 | 192.168.3.1 | 255.255.255.0 | N/A |
|  | S0/0/0 (DCE) | 10.3.3.2 | 255.255.255.252 | N/A |
|  | S0/0/1 | 10.2.2.1 | 255.255.255.252 | N/A |
| PC-A | NIC | 192.168.1.3 | 255.255.255.0 | 192.168.1.1 |
| PC-B | NIC | 192.168.2.3 | 255.255.255.0 | 192.168.2.1 |
| PC-C | NIC | 192.168.3.3 | 255.255.255.0 | 192.168.3.1 |

Objectives

Part 1: Build the Network and Verify Connectivity

Part 2: Configure EIGRP Routing

Part 3: Verify EIGRP Routing

Part 4: Configure Bandwidth and Passive Interfaces

Background / Scenario

Enhanced Interior Gateway Routing Protocol (EIGRP) is a powerful distance vector routing protocol and is relatively easy to configure for basic networks.

In this lab, you will configure EIGRP for the topology and networks shown above. You will modify bandwidth and configure passive interfaces to allow EIGRP to function more efficiently.

Note: The routers used with CCNA hands-on labs are Cisco 1941 Integrated Services Routers (ISRs) with Cisco IOS Release 15.2(4)M3 (universalk9 image). Other routers and Cisco IOS versions can be used. Depending on the model and Cisco IOS version, the commands available and output produced might vary from what is shown in the labs. Refer to the Router Interface Summary Table at the end of this lab for the correct interface identifiers.

Note: Make sure that the routers have been erased and have no startup configurations. If you are unsure, contact your instructor.

Required Resources

3 Routers (Cisco 1941 with Cisco IOS Release 15.2(4)M3 universal image or comparable)

3 PCs (Windows 7, Vista, or XP with terminal emulation program, such as Tera Term)

Console cables to configure the Cisco IOS devices via the console ports

Ethernet and serial cables as shown in the topology

Build the Network and Verify Connectivity

In Part 1, you will set up the network topology and configure basic settings, such as the interface IP addresses, device access, and passwords.

Cable the network as shown in the topology.

Configure PC hosts.

Initialize and reload the routers as necessary.

Configure basic settings for each router.

Disable DNS lookup.

Configure IP addresses for the routers, as listed in the Addressing Table.

Configure device name as shown in the topology.

Assign cisco as the console and vty passwords.

Assign class as the privileged EXEC password.

Configure logging synchronous to prevent console and vty messages from interrupting command entry.

Configure a message of the day.

Copy the running configuration to the startup configuration.

Verify connectivity.

The routers should be able to ping one another, and each PC should be able to ping its default gateway. The PCs will not be able to ping other PCs until EIGRP routing is configured. Verify and troubleshoot if necessary.

Configure EIGRP Routing

Enable EIGRP routing on R1. Use AS number 10.

R1(config)# router eigrp 10

Advertise the directly connected networks on R1 using the wildcard mask.

R1(config-router)# network 10.1.1.0 0.0.0.3

R1(config-router)# network 192.168.1.0 0.0.0.255

R1(config-router)# network 10.3.3.0 0.0.0.3

Why is it a good practice to use wildcard masks when advertising networks? Could the mask have been omitted from any of the network statements above? If so, which one(s)?

Solo debes anunciar las redes que controlas. En versiones anteriores de eigrp, se asumían límites con clase, lo que significa que todo el espacio de la red estaba anunciado.

Enable EIGRP routing and advertise the directly connected networks on R2 and R3.

You will see neighbor adjacency messages as interfaces are added to the EIGRP routing process. The messages on R2 are displayed as an example.

*Apr 14 15:24:59.543: %DUAL-5-NBRCHANGE: EIGRP-IPv4 10: Neighbor 10.1.1.1 (Serial0/0/0) is up: new adjacency

Verify end-to-end connectivity.

All devices should be able to ping each other if EIGRP is configured correctly.

Note: Depending on the operating system, it may be necessary to disable the firewall for the pings to the host PCs to be successful.

Verify EIGRP Routing

Examine the EIGRP neighbor table.

On R1, issue the show ip eigrp neighbors command to verify that the adjacency has been established with its neighboring routers.

R1# show ip eigrp neighbors

EIGRP-IPv4 Neighbors for AS(10)

H   Address                 Interface              Hold Uptime   SRTT   RTO  Q  Seq

(sec)         (ms)       Cnt Num

1   10.3.3.2                Se0/0/1                  13 00:24:58    8   100  0  17

0   10.1.1.2                Se0/0/0                  13 00:29:23    7   100  0  23

Examine the IP EIGRP routing table.

R1# show ip route eigrp

Codes: L - local, C - connected, S - static, R - RIP, M - mobile, B - BGP

D - EIGRP, EX - EIGRP external, O - OSPF, IA - OSPF inter area

N1 - OSPF NSSA external type 1, N2 - OSPF NSSA external type 2

E1 - OSPF external type 1, E2 - OSPF external type 2

i - IS-IS, su - IS-IS summary, L1 - IS-IS level-1, L2 - IS-IS level-2

ia - IS-IS inter area, * - candidate default, U - per-user static route

o - ODR, P - periodic downloaded static route, H - NHRP, l - LISP

+ - replicated route, % - next hop override

Gateway of last resort is not set

10.0.0.0/8 is variably subnetted, 5 subnets, 2 masks

D        10.2.2.0/30 [90/2681856] via 10.3.3.2, 00:29:01, Serial0/0/1

[90/2681856] via 10.1.1.2, 00:29:01, Serial0/0/0

D     192.168.2.0/24 [90/2172416] via 10.1.1.2, 00:29:01, Serial0/0/0

D     192.168.3.0/24 [90/2172416] via 10.3.3.2, 00:27:56, Serial0/0/1

Why does R1 have two paths to the 10.2.2.0/30 network?

EIGRP hace automáticamente el equilibrio de carga. R1 tiene dos formas de llegar a la red 10.2.2.0/30

Examine the EIGRP topology table.

R1# show ip eigrp topology

EIGRP-IPv4 Topology Table for AS(10)/ID(192.168.1.1)

Codes: P - Passive, A - Active, U - Update, Q - Query, R - Reply,

r - reply Status, s - sia Status

P 192.168.3.0/24, 1 successors, FD is 2172416

via 10.3.3.2 (2172416/28160), Serial0/0/1

P 192.168.2.0/24, 1 successors, FD is 2172416

via 10.1.1.2 (2172416/28160), Serial0/0/0

P 10.2.2.0/30, 2 successors, FD is 2681856

via 10.1.1.2 (2681856/2169856), Serial0/0/0

via 10.3.3.2 (2681856/2169856), Serial0/0/1

P 10.3.3.0/30, 1 successors, FD is 2169856

via Connected, Serial0/0/1

P 192.168.1.0/24, 1 successors, FD is 2816

via Connected, GigabitEthernet0/0

P 10.1.1.0/30, 1 successors, FD is 2169856

via Connected, Serial0/0/0

Why are there no feasible successors listed in the R1 topology table?

No se ha cumplido la function de confiabilidad

Verify the EIGRP routing parameters and networks advertised.

Issue the show ip protocols command to verify the EIGRP routing parameters used.

R1# show ip protocols

*** IP Routing is NSF aware ***

Routing Protocol is "eigrp 10"

Outgoing update filter list for all interfaces is not set

Incoming update filter list for all interfaces is not set

Default networks flagged in outgoing updates

Default networks accepted from incoming updates

EIGRP-IPv4 Protocol for AS(10)

Metric weight K1=1, K2=0, K3=1, K4=0, K5=0

NSF-aware route hold timer is 240

Router-ID: 192.168.1.1

Topology : 0 (base)

Active Timer: 3 min

Distance: internal 90 external 170

Maximum path: 4

Maximum hopcount 100

Maximum metric variance 1

Automatic Summarization: disabled

Maximum path: 4

Routing for Networks:

10.1.1.0/30

10.3.3.0/30

192.168.1.0

Routing Information Sources:

Gateway         Distance      Last Update

10.3.3.2              90      02:38:34

10.1.1.2              90      02:38:34

Distance: internal 90 external 170

Based on the output of issuing the show ip protocols command, answer the following questions.

What AS number is used?  10

What networks are advertised?  10.1.1.0/30, 10.3.3.0/30, 192.168.1.0/24

What is the administrative distance for EIGRP? 90 interna y 170 externa

How many equal cost paths does EIGRP use by default? 4

Configure Bandwidth and Passive Interfaces

EIGRP uses a default bandwidth based on the type of interface in the router. In Part 4, you will modify the bandwidth so that the link between R1 and R3 has a lower bandwidth than the link between R1/R2 and R2/R3. In addition, you will set passive interfaces on each router.

Observe the current routing settings.

Issue the show interface s0/0/0 command on R1.

R1# show interface s0/0/0

Serial0/0/0 is up, line protocol is up

Hardware is WIC MBRD Serial

Internet address is 10.1.1.1/30

MTU 1500 bytes, BW 1544 Kbit/sec, DLY 20000 usec,

reliability 255/255, txload 1/255, rxload 1/255

Encapsulation HDLC, loopback not set

Keepalive set (10 sec)

Last input 00:00:01, output 00:00:02, output hang never

Last clearing of "show interface" counters 03:43:45

Input queue: 0/75/0/0 (size/max/drops/flushes); Total output drops: 0

Queueing strategy: fifo

Output queue: 0/40 (size/max)

5 minute input rate 0 bits/sec, 0 packets/sec

5 minute output rate 0 bits/sec, 0 packets/sec

4050 packets input, 270294 bytes, 0 no buffer

Received 1554 broadcasts (0 IP multicasts)

0 runts, 0 giants, 0 throttles

1 input errors, 0 CRC, 0 frame, 0 overrun, 0 ignored, 1 abort

4044 packets output, 271278 bytes, 0 underruns

0 output errors, 0 collisions, 5 interface resets

4 unknown protocol drops

0 output buffer failures, 0 output buffers swapped out

12 carrier transitions

DCD=up  DSR=up  DTR=up  RTS=up  CTS=up

What is the default bandwidth for this serial interface? Depende la tarjeta del router puede variar.

How many routes are listed in the routing table to reach the 10.2.2.0/30 network? 2

Modify the bandwidth on the routers.

Modify the bandwidth on R1 for the serial interfaces.

R1(config)# interface s0/0/0

R1(config-if)# bandwidth 2000

R1(config-if)# interface s0/0/1

R1(config-if)# bandwidth 64

Issue show ip route command on R1. Is there a difference in the routing table? If so, what is it?

Codes: L - local, C - connected, S - static, R - RIP, M - mobile, B - BGP

D - EIGRP, EX - EIGRP external, O - OSPF, IA - OSPF inter area

N1 - OSPF NSSA external type 1, N2 - OSPF NSSA external type 2

E1 - OSPF external type 1, E2 - OSPF external type 2

i - IS-IS, su - IS-IS summary, L1 - IS-IS level-1, L2 - IS-IS level-2

ia - IS-IS inter area, * - candidate default, U - per-user static route

o - ODR, P - periodic downloaded static route, H - NHRP, l - LISP

+ - replicated route, % - next hop override

Gateway of last resort is not set

10.0.0.0/8 is variably subnetted, 5 subnets, 2 masks

C        10.1.1.0/30 is directly connected, Serial0/0/0

L        10.1.1.1/32 is directly connected, Serial0/0/0

D        10.2.2.0/30 [90/2681856] via 10.1.1.2, 00:03:09, Serial0/0/0

C        10.3.3.0/30 is directly connected, Serial0/0/1

L        10.3.3.1/32 is directly connected, Serial0/0/1

192.168.1.0/24 is variably subnetted, 2 subnets, 2 masks

C        192.168.1.0/24 is directly connected, GigabitEthernet0/0

L        192.168.1.1/32 is directly connected, GigabitEthernet0/0

D     192.168.2.0/24 [90/1794560] via 10.1.1.2, 00:03:09, Serial0/0/0

D     192.168.3.0/24 [90/2684416] via 10.1.1.2, 00:03:08, Serial0/0/0

- Después del cambio en el ancho de banda, solo hay una ruta que muestra la red 10.2.2.0/30 a través de 10.1.1.2 y S0 / 0/0.

Modify the bandwidth on the R2 and R3 serial interfaces.

R2(config)# interface s0/0/0

R2(config-if)# bandwidth 2000

R2(config-if)# interface s0/0/1

R2(config-if)# bandwidth 2000

R3(config)# interface s0/0/0

R3(config-if)# bandwidth 64

R3(config-if)# interface s0/0/1

R3(config-if)# bandwidth 2000

Verify the bandwidth modifications.

Verify bandwidth modifications. Issue a show interface serial 0/0/x command, with x being the appropriate serial interface on all three routers to verify that bandwidth is set correctly. R1 is shown as an example.

R1# show interface s0/0/0

Serial0/0/0 is up, line protocol is up

Hardware is WIC MBRD Serial

Internet address is 10.1.1.1/30

MTU 1500 bytes, BW 2000 Kbit/sec, DLY 20000 usec,

reliability 255/255, txload 1/255, rxload 1/255

Encapsulation HDLC, loopback not set

Keepalive set (10 sec)

Last input 00:00:01, output 00:00:02, output hang never

Last clearing of "show interface" counters 04:06:06

Input queue: 0/75/0/0 (size/max/drops/flushes); Total output drops: 0

Queueing strategy: fifo

Output queue: 0/40 (size/max)

5 minute input rate 0 bits/sec, 0 packets/sec

5 minute output rate 0 bits/sec, 0 packets/sec

4767 packets input, 317155 bytes, 0 no buffer

Received 1713 broadcasts (0 IP multicasts)

0 runts, 0 giants, 0 throttles

1 input errors, 0 CRC, 0 frame, 0 overrun, 0 ignored, 1 abort

4825 packets output, 316451 bytes, 0 underruns

0 output errors, 0 collisions, 5 interface resets

4 unknown protocol drops

0 output buffer failures, 0 output buffers swapped out

12 carrier transitions

DCD=up  DSR=up  DTR=up  RTS=up  CTS=up

Based on your bandwidth configuration, try and determine what the R2 and R3 routing tables will look like before you issue a show ip route command. Are their routing tables the same or different?

La tabla de enrutamiento R2 será la misma que antes. Todavía tendrá 2 rutas de igual costo para la red 10.3.3.0/30. La tabla de enrutamiento R3 ahora solo tendrá 1 ruta a la red 10.1.1.0/30 a través de R2

Configure G0/0 interface as passive on R1, R2, and R3.

A passive interface does not allow outgoing and incoming routing updates over the configured interface. The passive-interface interface command causes the router to stop sending and receiving Hello packets over an interface; however, the network associated with the interface is still advertised to other routers through the non-passive interfaces. Router interfaces connected to LANs are typically configured as passive.

R1(config)# router eigrp 10

R1(config-router)# passive-interface g0/0

R2(config)# router eigrp 10

R2(config-router)# passive-interface g0/0

R3(config)# router eigrp 10

R3(config-router)# passive-interface g0/0

Verify the passive interface configuration.

Issue a show ip protocols command on R1, R2, and R3 and verify that G0/0 has been configured as passive.

R1# show ip protocols

*** IP Routing is NSF aware ***

Routing Protocol is "eigrp 10"

Outgoing update filter list for all interfaces is not set

Incoming update filter list for all interfaces is not set

Default networks flagged in outgoing updates

Default networks accepted from incoming updates

EIGRP-IPv4 Protocol for AS(10)

Metric weight K1=1, K2=0, K3=1, K4=0, K5=0

NSF-aware route hold timer is 240

Router-ID: 192.168.1.1

Topology : 0 (base)

Active Timer: 3 min

Distance: internal 90 external 170

Maximum path: 4

Maximum hopcount 100

Maximum metric variance 1

Automatic Summarization: disabled

Maximum path: 4

Routing for Networks:

10.1.1.0/30

10.3.3.0/30

192.168.1.0

Passive Interface(s):

GigabitEthernet0/0

Routing Information Sources:

Gateway         Distance      Last Update

10.3.3.2              90      00:48:09

10.1.1.2              90      00:48:26

Distance: internal 90 external 170

Reflection

You could have used only static routing for this lab. What is an advantage of using EIGRP?

EIGRP Se ajusta automaticamente a los cambios que pueden presentarse en una red

Router Interface Summary Table

| Router Interface Summary |  |  |  |  |
| --- | --- | --- | --- | --- |
| Router Model | Ethernet Interface #1 | Ethernet Interface #2 | Serial Interface #1 | Serial Interface #2 |
| 1800 | Fast Ethernet 0/0 (F0/0) | Fast Ethernet 0/1 (F0/1) | Serial 0/0/0 (S0/0/0) | Serial 0/0/1 (S0/0/1) |
| 1900 | Gigabit Ethernet 0/0 (G0/0) | Gigabit Ethernet 0/1 (G0/1) | Serial 0/0/0 (S0/0/0) | Serial 0/0/1 (S0/0/1) |
| 2801 | Fast Ethernet 0/0 (F0/0) | Fast Ethernet 0/1 (F0/1) | Serial 0/1/0 (S0/1/0) | Serial 0/1/1 (S0/1/1) |
| 2811 | Fast Ethernet 0/0 (F0/0) | Fast Ethernet 0/1 (F0/1) | Serial 0/0/0 (S0/0/0) | Serial 0/0/1 (S0/0/1) |
| 2900 | Gigabit Ethernet 0/0 (G0/0) | Gigabit Ethernet 0/1 (G0/1) | Serial 0/0/0 (S0/0/0) | Serial 0/0/1 (S0/0/1) |
| Note: To find out how the router is configured, look at the interfaces to identify the type of router and how many interfaces the router has. There is no way to effectively list all the combinations of configurations for each router class. This table includes identifiers for the possible combinations of Ethernet and Serial interfaces in the device. The table does not include any other type of interface, even though a specific router may contain one. An example of this might be an ISDN BRI interface. The string in parenthesis is the legal abbreviation that can be used in Cisco IOS commands to represent the interface. |  |  |  |  |